s2spice.c - A program to create a Pspice model from Maxwell Eminence results

By Salvador Iranzo ( salvador.iranzo@cern.ch)

1 - Sep - 1997

Introduction

The s2spice.c program reads a file containing S-parameter data and creates a Pspice subcircuit. The input file can be created saving Maxwell Eminence or HFSS results in Touchstone format, then, after running s2spice we obtain a .lib file that can later be used to create a Pspice equivalent circuit. The complete procedure is explained in the Maxwell WWW pages. The format used for the model is explained in the following Application Note from Microsim:

“ Create S-parameter subcircuits for microwave and RF applications”

available in the manuals or through the WWW. This note only contemplates the 2-port case. The s2spice program generalises the method up to eight ports.

The program has been compiled for the AIX operating system and is available at:

/afs/cern.ch/project/parc/maxwell

Syntax

The program can be invoked in two different formats:

s2spice touchstone_file

or

s2spice touchstone_file Z1 Z2 [ Z3 ... Z8 ]

In the first case the program will ask for the number of ports and the input impedance will be set to the value given in the line marked with # in the input file. Observe that this is the impedance of port number 1 for the lowest frequency solved.

In the second case the input impedances are given for every port, and there is no need to ask for the number of ports.

If the program is called without arguments it prints some help lines.

Program listing

#include <stdio.h>

#include <math.h>

#define MAXLINE 500

#define MAXFREQS 200

#define MAXPORTS 8

#define MAXNAME 16

/* help; prints help */

void help()

{

printf("Usage: s2spice Touchstone_file [ Z1 Z2 [ Zi... ] ]\n");

printf("This program obtains the Spice equivalent circuit from the S-parameters\n");

printf("The number of ports must be between 1 and 8\n");

}

/* copy: copies 'num' characters from vector to output file */

/* if num==0 copies until \n is found */

void copy(char s[],FILE *ofp,int num,int add)

{

int i,c;

for (i=0;s[i]!='\n' && ((i<num) || (num==0));i++)

putc(s[i],ofp);

if (add) putc('\n',ofp);

}

void error(int c)

{

switch (c) {

case 1: printf("Number of ports must be between 1 and 8\n");

case 2: printf("# not found in input file\n");

}

}

double absol(double c)

{

if (c<0) {

c=-c;

return c;

} else {

return c;

}

}

int sgn(double c)

{

if (c<0) {

return -1;

} else {

return 1;

}

}

void main(int argc, char *argv[])

{

int c,i,j,f;

int ports;

int funits;

int cont;

int numf;

char line[MAXLINE]=".SUBCKT ";

char line2[MAXLINE]=" 1 2 3 4 5 6 7 8 9 \n";

char lib[MAXNAME]="lib\0            ";

FILE *in,*out;

double z[MAXPORTS];

double ph,prevph,offset,mag;

double s[MAXFREQS][MAXPORTS][MAXPORTS][2];

double freqs[MAXFREQS];

char name[MAXNAME];

/* invoke help */

/* the program must be called with 2,4...10 arguments */

if ((argc < 2) || (argc > 10) || (argc == 3)) {

help();

goto end;

}

/* obtain number of ports */

if (argc == 2) {  /* the Z's are not specified in the command line */

printf("Number of ports of the structure\n");

printf("Return for help\n");

if ((ports=getchar())=='\n') {

help();

goto end;

}

/* decode number of ports */

ports=ports-'0';

if ((ports == 0) || (ports > MAXPORTS)) {

error(1);

goto end;

}

} else {  /* the Z's are given in the command line */

ports=argc-2;

}

printf("The circuit has %d ports\n",ports);

/* open input file */

i=0;

while ((name[i]=*argv[1]) != '\0') {

i++;

argv[1]++;

}

if ((in=fopen(&name,"r")) == NULL) {

printf("Can't open file\n");

goto end;

}

/* open output file. Same name as input but with .lib extension */

while (name[i] != '.') i--;

j=0;

i++;

while (MAXNAME >= (i+j)) {

name[i+j]=lib[j];

j++;

}

if ((out=fopen(&name,"w")) == NULL) {

printf("Can't open file\n");

goto end;

}

/* build first line of output file */

copy(line,out,8,0);

copy(name,out,i-1,0);

copy(line2,out,3+ports*2,1);

/* copy comment lines from input to output files */

while ((c=getc(in))=='!'){

line[0]='*';

for (i=1;((line[i]=getc(in))!='\n') && (i<MAXLINE);i++);

copy (line,out,MAXLINE,1);

}

/* read and decode format line */

while ((c!='#') && (c!=EOF))

c=getc(in);

if (c==EOF){

error(2);

goto end;

} else {

line[0]='*';

for (i=1;((line[i]=getc(in))!='\n') && (i<11);i++);

copy (line,out,12,1);

}

if (line[2]=='M') { /* Frequencies in MHz  */

funits=1000;

} else {

funits=1;

}

/* input impedances */

fscanf(in,"%lf",&z[1]);

for (i=1;i<=ports;i++) {

if (argc==2) {  /* takes the Z value from the input file */

z[i]=z[1];

} else {        /* takes the Z value from the command line */

sscanf(argv[1+i],"%lf",&z[i]);

}

}

/* copies Z's to output file as comments */

fprintf(out,"*");

for (i=1;i<=ports;i++) fprintf(out," Z%d=%7.6f",i,z[i]);

fprintf(out,"\n\n");

/* define resistances for Spice model */

for (i=1;i<=ports;i++) {

fprintf(out,"R%dN %d %d %e\n",i,i,10*i,-z[i]);

fprintf(out,"R%dP %d %d %e\n",i,10*i,10*i+1,2*z[i]);

}

fprintf(out,"\n");

/* read S parameters into matrix */

cont=1;

for (f=1;cont;f++) {

fscanf(in,"%lf",&freqs[f]);

if (ports!=2) {

for (i=1;i<=ports;i++)

for (j=1;j<=ports;j++) {

fscanf(in,"%lf",&s[f][i][j][1]);

fscanf(in,"%lf",&s[f][i][j][2]);

}

} else { /* the input format is different for two ports */

fscanf(in,"%lf",&s[f][1][1][1]);

fscanf(in,"%lf",&s[f][1][1][2]);

fscanf(in,"%lf",&s[f][2][1][1]);

fscanf(in,"%lf",&s[f][2][1][2]);

fscanf(in,"%lf",&s[f][1][2][1]);

fscanf(in,"%lf",&s[f][1][2][2]);

fscanf(in,"%lf",&s[f][2][2][1]);

fscanf(in,"%lf",&s[f][2][2][2]);

}

/* discard commented lines */

while (((c=getc(in))!='\n') && (c!=EOF));

while ((c=getc(in))=='!')

while ((c=getc(in))!='\n');

while ((c == ' ') || (c == '\n')) c=getc(in); /* discard blank lines at the end */

if (c==EOF) {

cont=0;

numf=f;

} else {

c=ungetc(c,in);

}

}

printf("%d frequency values read\n",numf);

/* write values to output file*/

for (i=1;i<=ports;i++) {

for (j=1;j<=ports;j++) {

fprintf(out,"*S%d%d FREQ DB PHASE\n",i,j);

if (j==ports) {

fprintf(out,"E%d%d %d%d %d FREQ {V(%d,%d)}=\n",i,j,i,j,ports+1,10*j,ports+1);

} else {

fprintf(out,"E%d%d %d%d %d%d FREQ {V(%d,%d)}=\n",i,j,i,j,i,j+1,10*j,ports+1);

}

offset=0;

prevph=0;

for (f=1;f<=numf;f++) {

mag=20*log10(s[f][i][j][1]);

ph=s[f][i][j][2];

if ((absol(ph-prevph))>180) {

offset=offset-360*sgn(ph-prevph);

}

prevph=ph;

fprintf(out,"+(%14egHz,%14e,%14e)\n",freqs[f]*funits,mag,ph+offset);

}

fprintf(out,"\n");

}

}

fprintf(out,".ENDS\n");

c=fclose(in);

c=fclose(out);

end:

printf("Processing Completed\n");

}

Program notes

To compile the program execute:

cc s2spice.c -lm

to link it with the math library.

The maximum number of frequency values is set to 200 at the moment.

The program flow is the following:

After declaring all variables the help routine is invoked if the number of arguments is not correct ( 1, 3 or more than 10 arguments). In this case it exits.

If the number of arguments is 2 it prompts for the number of ports of the structure, and checks that it is between 2 and 8. If the input impedances are given in the command line, the number of ports is given by the number of values given. It then prints the number of ports on the screen.

The next step is to open the input and the output files. The output file has the same name as the input file but with the .lib extension.

Now, it writes the first line of the output file with the following format:

.SUBCKT “name” 1 2 [ 3 ... 8 ]

and copies the comment lines from the input to the output, until it finds the “ # “ character. This line specifies the frequency units which can be either MHz or GHz.

The next step is to obtain the input impedances. If the number of arguments is 2 the input impedance for all ports is the same and is taken from the line starting with # of the input file. If not, they are taken from the command line. Now it prints the output impedance of every port to the output file.

The RxN and RxP resistances are written to the output file. For more details on the S-parameter subcircuit see the Microsim application note.

Here the main part of the program comes. The S-parameters are read from the input file into a matrix. Note that the format is different for 2-port structures because all matrix values come in the same line. It reads the S-matrix for one frequency value and advances until the values for the next frequency start and loops again. When it finds the “EOF” mark, prints the number of  frequencies/matrices read on the screen so that one can check that everything is right!

Finally the output file is written according to the format explained in the application note. For every parameter it writes the following line:

*Sxx FREQ DB PHASE

and the table with the amplitude and phase values at different frequencies that specify the behaviour of the voltage controlled voltage source that simulates the response of this parameter. The phase wrapping is very important because it sets the delay of the signals through the structure. When all the S parameters are written the .ENDS tag is added to the file, the files are closed and the program exits.

Notes

When the model is used in a Pspice transient simulation some problems may appear. Here, there are some tips to obtain better results.

- Non-causal response:

If this warning appears it is possible to add a delay to the S-parameters in the .lib file. To know how to add a delay to a voltage controlled voltage source read the Pspice manuals. Another possibility is to perform the transient analysis for a longer time, until the response is zero, as the values at time ‘t’ might affect the result at previous times.

- Low frequencies:

You can add the values for low frequencies by hand to the .tch or .lib files if the input signal has important components for this frequencies and they have not been solved by Maxwell. You will normally know the response of the structure for DC.

- Joining data from various frequency sweeps:

The files must be joined while in .tch format so that the phase wrapping is correctly calculated.